# AVCaptureQuad

# Overview

AVCaptureQuad calls the MWCaptureVideoFrameToVirtualAddressEx interface of capture card to create multi-screen overlays, with up to four screens. The location and size of each screen can be set in the configuration files.

# Usage

## 2.1 Compile

AVCaptureQuad is developed and compiled using Qt (version 5.5 and above), and displayed the images using OpenGL.

Install Qt

The general procedure of Qt installation is as follows, for detailed steps, please search the tutorial on the internet.

- Choose and download a proper version from http://download.qt.io/archive/qt/. Here takes the Qt 5.5 as an example, visit the downloads page http://download.qt.io/archive/qt/5.5/5.5.0/, download the installation file depending on your system. (It is recommended to download the Qt 5.5, if you are using 32-bit Linux.)

- Adjust permission. After downloading, right click the file to check and adjust the file permission, and then run the installer and follow the instructions to complete the installation.

Install OpenGL

- Ubuntu

Open a terminal and execute the following commands to install the OpenGL in ubuntu.

sudo apt-get install build-essential

sudo apt-get install libgl1-mesa-dev

sudo apt-get install libglu1-mesa-dev

sudo apt-get install libglut-dev

Note: Run sudo apt-get install freeglut3-dev instead of sudo apt-get install libglut-dev, if it doesn’t work. For other platforms, please search the associated tutorials on the Ethernet.

- CentOS

Open a terminal and execute the following command to install the OpenGL in CentOS.

yum install mesa-libGL-devel

Configure a compiler

Launch Qt Creator, Click Open Project and choose AVCaptureQuad.pro, then Click Build & Run to configure a compiler.

As a result, the executable file is generated in the bin directory.

## 2.2 Run AVCaptureQuad

Run AVCaptureQuad by executing the following command in the bin directory.

./AVCaptureQuad

As a result, there would be 4 configuration files generated in the directory where the executable file is located after running.

Each configuration file contains three parts: channel configurations, pro capture cards information, and parameter ranges.

For example:

index:0 posx:0 posy:0 poswid:50 poshei:50 layer:1

index:1 posx:50 posy:0 poswid:50 poshei:50 layer:2

index:2 posx:0 posy:50 poswid:50 poshei:50 layer:3

index:-1 posx:50 posy:50 poswid:50 poshei:50 layer:4

********************************************************

index:0 name:Pro Capture Dual SDI 01-0

index:1 name:Pro Capture Dual SDI 01-1

index:2 name:Pro Capture HDMI 4K 04-0

********************************************************

posx 0-99

posy 0-99

poswid 1-100

poshei 1-100

layer 1-4

- Channel Configurations

index:0 posx:0 posy:0 poswid:50 poshei:50 layer:1

- “index” indicates the open channel index. The index must not be duplicated or exceed the card’s capability.

- “posx” indicates the x coordinate of image output by the channel on the entire screen. The value is a percentage multiplied by 100, and the actual location is the width of display pixel x posx/100.

- “posy” indicates the y coordinate of image output by the channel on the entire screen. The value is a percentage multiplied by 100, and the actual location is the height of display pixel x posy/100.

- “poswid” indicates the display width of image output by the channel. The value is a percentage multiplied by 100, and the actual width equals the display pixel width ×poswid/100.

- “poshei” indicates the display height of image output by the channel. The value is a percentage multiplied by 100, and the actual height equals the display pixel width x poshei/100.

- “layer” indicates the display relationship between the images. 4 indicate the image will be displayed at the top, if there is a cross area between the images, the layer with larger number would cover the one with a smaller number.

- Pro Capture Cards Information

index:0 name:Pro Capture Dual SDI 01-0

- “index” indicates the device channel index.

- “name” indicates the device name. “Pro Capture Dual SDI” indicates the product name, 01 indicates the board index, and 0 indicates the channel index.

- Parameter Ranges

- “posx” indicates the x coordinate of the image, the range of value is 0-99.

- “posy” indicates the y coordinate of the image, the range of value is 0-99.

- “poswid” indicates that the width of image, the range of value is 1-100.

- “poshei” indicates the height of image, the range of value is 1-100.

- “layer” indicates the layer of images, the range of value is 1-4.

- Only the Channel Configurations part can be modified in the configuration files, the pro cards information and the parameter ranges parts are used for references.

There are five buttons on the application interface, START SETTING 1, START SETTING 2, START SETTING 3, START SETTING 4 and STOP. For example, pressing START SETTING 1 will load setting1.cfg file, open the capture channel, display overly according to the size and position of the pictures set in configuration files. The STOP button will stop the capture process.

# Working Environment

Pro capture family cards support AVCaptureQuad with their drivers installed.